MPM INFOSOFT PVT. LTD.

AUDIO ALERT & INFORMATION SYSTEM

Phase 2 — Deliverables (Bullet Points)

# Phase 1 — Completed

Phase 1 Deliverables

1.1 Translation & Text-to-Speech (TTS) Pipeline

- Translation using Gemini API and text-to-speech generation using VoiceMaker TTS have been successfully implemented.

- Multiple translation engines (NLLB-600M, Argos Translate, IndicTrans) and TTS engines (Piper TTS, IndicTTS, Sarvam AI, VoiceMaker) were evaluated.

- Gemini API + VoiceMaker TTS were finalized based on voice quality, regional language support, reliability, response time, and cost.

1.2 Cloud-to-Edge Alert Processing

- End-to-end alert flow has been validated from the cloud database to edge nodes.

- The CM5 gateway continuously polls the cloud database for new alerts, translates the alert message into the configured regional language, generates the corresponding speech, and transmits the audio file over Wi-Fi to the designated edge node (RPi4) for playback.

- Generated audio files are cached locally on the gateway. If an identical alert is received again, the cached audio is reused, eliminating the need for repeated translation and TTS generation.

1.3 Priority-Based Multi-Zone Audio Playback

- Alert audio playback is managed using a priority queue on each edge node, ensuring higher-priority alerts are played before lower-priority alerts.

- The CM5 gateway supports simultaneous delivery of alerts to multiple edge nodes across different plant zones with reliable and low-latency communication.

- Alert audio continues to repeat until the corresponding acknowledgement is registered in the cloud database.

1.4 Configuration Dashboard

- A centralized configuration dashboard has been developed to support plant-wise deployment and management.

- Features include:

  - Configuration of multiple production lines.

  - Configuration of multiple zones within each line.

  - Assignment and management of edge nodes for each zone.

  - Regional language selection for individual zones.

1.5 Alert Logging

- Comprehensive alert logs are maintained within the gateway.

- Logged information includes alert text, alert timestamp, priority, playback status, etc..

# Phase 2 — Deliverables

## D1: Central Command Center Dashboard

- Web-based control panel hosted on CM5, accessible via any browser on the local network

- Real-time zone status display (online/offline)

- Announcement history log

- Edge node health: CPU temperature, uptime, audio service status (if playing any audio at the moment)

- Manual broadcast panel: select zone, choose language, and trigger announcement on demand

- Manual broadcast supports two audio modes:

  - Dynamic Text — type a message, select a language for translation, system generates TTS audio and broadcasts it

  - Pre-Recorded File — upload or select from a library of pre-recorded audio file and broadcast directly

## D2: Live Voice Paging

- Push-to-talk voice paging from the command center dashboard

- Operator selects target zone (or all zones), presses PTT button, speaks into microphone

- Voice streamed to selected RPi4 edge node(s)

- Functions like an industrial PA system

- Supports single-zone, zone-group, and plant-wide paging

- Enables direct human communication to any part of the foundry

- Useful for instructions, coordination, and supervisor-to-floor communication

## D3: Scheduled Announcements

- Automated scheduling engine — broadcasts pre-configured announcements at set times without manual intervention.

- Covers: shift start/end reminders, lunch breaks, Periodic Safety Reminders, etc..

- Configured through the command center dashboard.

- Each scheduled announcement supports two audio modes:

  - Dynamic Text — enter message text by selecting language for translation, system generates TTS audio and caches it at schedule creation time for instant playback

  - Pre-Recorded File — upload or select a pre-recorded audio file to be played at the scheduled time

- Each scheduled announcement can target a specific zone, zone group, or entire plant

## D4: SOP Step-by-Step Audio Guidance

- SOPs stored as ordered sequences of steps; audio system walks the operator through each step one at a time

- Full CRUD for SOPs and individual steps via the dashboard:

  - Create — add new SOPs and define steps in order

  - Read — view all SOPs and their step sequences

  - Update — edit step content, reorder steps, change step type

  - Delete — remove individual steps or entire SOPs

- Each step can be configured as one of two types:

  - Dynamic Text — enter message text, select language for translation; system generates TTS audio at runtime

  - Pre-Recorded Voice — upload or select a pre-recorded audio file (MP3/WAV) to be played for that step

- Initiation: manual from dashboard

- System announces the first step to the relevant zone, then waits for acknowledgement before announcing the next step

- Acknowledgement methods: dashboard button

- If a step is not acknowledged within configurable timeout, the system repeats the announcement

- Use cases: furnace startup, mould changeover, new operator training, etc..

- Full audit trail: every step execution and acknowledgement logged with operator ID and timestamp

Technologies

- MQTT (over Wi-Fi/Ethernet)

- HTTP (over Wi-Fi/Ethernet)

- Websockets